| BR 960 005 | ClassicLine mobile tank eyewash station with stainless steel tank |
| --- | --- |
| ClassicLine mobile tank eyewash station with stainless steel tank - for mobile use or when no water connection is available - tank of stainless steel, with a capacity of 18 litres, for a filling volume of water of approx. 14 litres, on trolley with solid rubber tyres - water inlet via large flap lid on the tank, Tamper-proof design due to screw connection - pressure build-up via compressed air with a filling pressure of 6 bar, compressed air connection tyre inflation valve - integrated pressure relief valve that opens at approximately 6 bar filling pressure - pressure loss of approx. 0.2 bar after one month - actuation by activation of the hand-held eye shower - hand-held eye shower with one spray head, mounting by wall bracket on the tank - ergonomically shaped handle with integrated, adjusting activation made of plastics, valve not self-closing - high-performance spray head for large-scale dispersion of water, with plastic spray plate, largely scale-free, with rubber sleeves and sealed dust caps - integrated automatic flow regulator 7 litres / minute (1.85 GPM) for a flush time of approximately 2 minutes - integrated backflow preventer - stainless steel covered hose, length 1.5 meter, water inlet ½” female, DIN-DVGW tested and certificated - sign for eye shower according to EN ISO 7010 und ASR A1.3 mounted on the tank, self-adhesive PVC-film, 150 x 150 mm, viewing distance 15 metre - dimensions (H x W x D): 850 x 450 x 450 mm - according to ANSI Z358.1-2014 and EN 15154-4:2009 Manufacturer: B-SAFETY or equal Article-No.: BR 960 005 Technical Specifications Filling volume: 14 litre Filling pressure: 6 bar Flow rate: 7 l/min (1.85 GPM) Flush time: about 2 minutes Water filling: Flap lid (secured by screw connection) Compressed air connection: tyre inflation valve Dimensions (H x W x D): 850 x 450 x 450 mm Product Overview BR 960 005: Flow rate 7 l/min (1,85 GPM) - Flush time about 2 minutes |  |